## LAN Network Traffic Monitoring and Analysis Dashboard

### 1. What exactly is being monitored?

The system monitors traffic inside a Local Area Network (LAN), not the internet provider itself.

Specifically, it monitors:

- Devices connected to the LAN (PCs, phones)

- Bandwidth usage per device

- Network protocols and applications (e.g. streaming, downloads)

- Traffic patterns over time (peak hours, abnormal usage)

In simple words:
“We monitor who is using the network, how much data they use, and what type of traffic they generate.”

### 2. Existing System (Current Situation)

In most institutions today:

- A basic router (TP-Link, Cisco, etc.) connects devices to the network

- The router provides:

  - Total bandwidth usage

  - Simple logs

- No real-time, detailed visibility

Problems with the existing system:

- Cannot see which device is consuming most bandwidth

- No per-application visibility

- Difficult to detect misuse or abnormal traffic

- Limited or short-term logs

- Decisions are reactive, not proactive

This is what your left-side drawing represents:

Devices connected → router → very limited information

### 3. Proposed System (Your Solution)

Your proposed system adds a monitoring layer between the LAN and the administrator.

This matches your right-side drawing:

- Monitored Network

- Monitoring System

- Communication via SNMP / packet capture

### 4. Main Components of the Proposed System

#### A. Monitored Network

This is the existing LAN:

- Switches

- Routers

- Connected devices (users)

No major change here.

#### B. Traffic Collection Layer

This is new and very important.

It collects data using:

- SNMP → device statistics (bandwidth, interfaces)

- Packet capture / flow data → traffic behavior

- Router or switch mirroring (if available)

Purpose:

- Capture real-time traffic data

- Identify devices and usage patterns

#### C. Monitoring & Analysis Server

This is the core of your project.

It:

- Receives raw traffic data

- Processes and analyzes it

- Stores it in a database

- Calculates:

  - Bandwidth per device

  - Bandwidth per application

  - Usage over time

#### D. Dashboard & Visualization Layer

This is what the admin sees.

It provides:

- Real-time dashboards

- Charts and graphs

- Tables and reports

Examples:

- Line chart: bandwidth usage over 24 hours

- Pie chart: bandwidth usage per device

- Table: top 10 bandwidth-consuming devices

### 5. How the System Works (Simple Flow)

- Devices generate traffic on the LAN

- Traffic data is collected via SNMP / monitoring tools

- Data is sent to the monitoring server

- Server analyzes and stores the data

- Dashboard displays results in real time

- Alerts are triggered if abnormal usage is detected

### 6. Why This System Is Better

| Feature | Existing System | Proposed System |
| --- | --- | --- |
| Visibility | Total usage only | Per device & per app |
| Monitoring | Manual | Real-time |
| Alerts | None or basic | Automated |
| Reports | Short-term logs | Long-term analytics |
| Decision Making | Guesswork | Data-driven |

## Tools & Technology Stack (Summary)

### Hardware & Network

- Computers / PCs – Generate LAN traffic

- Router / Switch – Connect network devices

### Traffic Monitoring

- Wireshark – Captures and analyzes network traffic

- SNMP – Collects device bandwidth statistics

### Backend & Server

- PHP – Data processing and analysis

- XAMPP – Local server (Apache, PHP, MySQL)

- MySQL – Stores traffic data and reports

### Frontend & Dashboard

- HTML – Page structure

- CSS – User interface design

### Users

- System Administrator – Monitors, analyzes, and manages the network

### 8. One-Sentence Summary

“This project introduces a real-time LAN traffic monitoring dashboard that gives administrators clear visibility into device-level and application-level bandwidth usage, enabling better control, performance optimization, and proactive network management.”